SECTION I - GENERAL INFORMATION

Chemical Name   5  Synonyms

N/A

Trade  Name   &   Synonyms LIQUI-CELLERATE,    O/G

tical   Family:

AMMONIUM  NITRATE   SOLUTION

Formula  Mixture   —>   X

Manufacturer's  Name:

MANTEK,    DIVISION   OF   NCH   CORP.

Addres s:

BOX  152170

IRVING,	TEXAS   75015

Prepared  By:

L Boynton/Chemist

Product  Code Number 9390

ergency  Phone Number 800-424--9300

SECTION II - HAZARDOUS INGREDIENTS THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

Chemical Name (IngireHi ents)

AMMONIUM NITRATE

UREA

IRRITANT IRRITANT

TLV

N/E   1.

N/E   1.

PEL

N/E   2.

N/E   2.

STEL

N/E N/E

6484-52-2 57-13-6

SECTION III - PHYSICAL DATA

Boiling  Point   (F) :	225°	Specific   Gravity   (H20=l) :	1.326

Vapor   Pressure    (MM HG) :	16	Color:	YELLOW

Vapor  Density   (Air=l) :	<1	Odor:	AMMONIA

PH  @   100%    :	7.5	Clarity:	TRANSPARENT

% Volatile  by Volume:	17	Evaporation Rate   (BU A/C=1) :	<1

H2O   Solubility:	COMPLETE	Viscosity:	NON-VISCOUS

SECTION IV - FIRE AND EXPLOSION HAZARD

Flash Point	Flammable Limits	LEL	UEL

NON-FLAM / N/A	N/A	N/A	N/A

Extinguishing Media

<—Foam	<--Alcohol Foam	<—CO2         <—Dry Chemical        X <—Water Spray      <--Other

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR. COOL FIRE-EXPOSED CONTAINERS WITH WATER SPRAY TO PREVENT

BURSTING.

Unusual Fire and Explosion Hazards: NONE KNOWN.

Aerosol Level (NFPA 30B):    N/A

NFPA 704 Hazard Rating        (0=Insignificant l=Slight  2=Moderate  3=High  4=Extreme) 1  <--Health       0  <--Flammability  0  <—Instability      <—Special

SECTION V - HEALTH HAZARD DATA

Threshold Limit Value: NOT ESTABLISHED.

Effects of Overexposure:

-Acute (Short Term Exposure)

EYE CONTACT: MAY CAUSE IRRITATION SEEN AS TEARING AND REDNESS.

SKIN CONTACT: MAY CAUSE IRRITATION SEEN AS ITCHING AND REDNESS UPON PROLONGED OR REPEATED CONTACT. INHALATION: MAY CAUSE RESPIRATORY IRRITATION SEEN AS COUGHING AND SNEEZING.

INGESTION: MAY CAUSE KIDNEY EFFECTS AND BLOOD DISORDERS (METHEMOGLOBINEMIAJ IN INFANTS, WHICH IS A LOSS OF EFFICIENCY IN THE RED BLOOD CELLS AND EVIDENCED BY BLUE SKIN. MAY CAUSE CENTRAL NERVOUS SYSTEM EFFECTS SUCH AS HEADACHE, DIZZINESS, WEAKNESS AND NAUSEA.

-Chronic (Long Term Exposure) NO HUMAN CHRONIC EFFECTS KNOWN-MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE ARE PRE-EXISTING RESPIRATORY AND SKIN CONDITIONS SUCH AS ASTHMA, EMPHYSEMA AND DERMATITIS. TARGET ORGANS: CENTRAL NERVOUS SYSTEM AND KIDNEYS- THE PRIMARY ROUTES OF EXPOSURE ARE SKIN AND EYE CONTACT.

Primary Routes of Entry:        <--Inhalation     X <—Ingestion         <--Absorption

,ency and First Aid Procedures:

-Inhalation: REMOVE FROM THE AREA TO FRESH AIR. SEEK MEDICAL ATTENTION IF RESPIRATORY IRRITATION DEVELOPS OR IF BREATHING BECOMES DIFFICULT.

-Eye Contact: RINSE THE EYES WITH WATER- REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING WITH PLENTY OF WATER FOR SEVERAL MINUTES. SEEK MEDIUM. ATTENTION IF

MATERIAL SAFETY DATA SHEET: LIQUI-CELLERATE, O/G SECTION V - HEALTH HAZARD DATA (Continued)

IRRITATION DEVELOPS.

-Skin Contact: WASH AFFECTED AREAS WITH PLENTY OF SOAP AND WATER FOR SEVERAL MINUTES- SEEK MEDICAL ATTENTION IF IRRITATION DEs/ELOPS,

-Ingestion:
GIVE 3 TO 4 GLASSES OF WATER, BUT DO NOT INDUCE VOMITING. IF VOMITING OCCURS, GIVE FLUIDS AGAIN. SEEK MEDICAL ATTENTION IF DISCOMFORT OCCURS,	^-^

-Notes to Physician: THERE IS NO SPECIFIC ANTIDOTE. TREAT THE PATIENT SYMPTOMAIICALLY.

SECTION VI - TOXICITY INFORMATION

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:

IARC—> No	NTP—> No	OSHA—> No	ACGIH--> No	OTHER—> No

PRODUCT MIXTURE ORL-RAT LD50: >2000 MG/KG   3.

AMMONIUM NITRATE ORL-RAT LD50: 2217 MG/KG   4. ORL-RAT TDLo: 65 MG/KG/26W-I   4.

UREA

ORL-RAT LD50: 8471 MG/KG   4. IHL-RAT TCLo: 288 MG/M3/17W-I    4. SKN-HMN SDT: 20%/24H MODERATE   4.

SECTION VII - REACTIVITY DATA

Stability:         X <—Stable      <—Unstable

Conditions to Avoid:

DO NOT EVAPORATE TO DRYNESS AS EXPLOSIVE AMMONIUM NITRATE WILL RESULT -

Incompatibility (Materials to Avoid):

STRONG OXIDISING AGENTS SUCH AS CHLORINE BLEACH AND CONCENTRATED HYDROGEN PEROXIDE; WATER-REACTIVE MATERIALS SUCH AS OLEUM; BRASS, BRONZE OR OTHER

COPPER CONTAINING ALLOYS OR GALVANISED METALS. UNDER CERTAIN CONDITIONS, NITRITES MAY REACT WITH SECONDARY AMINES TO FORM CARCINOGENIC NITROSAMINES.

Hazardous Decomposition Products:

OXIDES OF NITROGEN; AMMONIA AND HYDROGEN SULFIDE.

Hazardous Polymerization:	<--May Occur      X <--Will Not Occur

Conditions to Avoid:

N/A

SECTION VIII - SPILL OR LEAK PROCEDURES

Steps to be Taken if Material is Released or Spilled:

WEAR APPROPRIATE PROTECTIVE CLOTHING. USE CARE AS SPILLS MAY BE SLIPPERY- DIKE AND CONTAIN SPILL. ABSORB WITH AN INERT MATERIAL, AND TRANSFER ALL

MATERIAL INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL, FLUSH AREA WITH WATER.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE AND LOCAL REGULATIONS.

Neutralizing Agent: N/A

SECTION IX - SPECIAL PROTECTION INFORMATION

Required Ventilation:

GENERAL VENTILATION IS NORMALLY ADEQUATE. LOCAL VENTILATION IS RECOMMENDED TO CONTROL EXPOSURE FROM OPERATIONS THAT CAN GENERATE MISTS.

Respiratory Protection:

A NIOSH/MSHA APPROVED RESPIRATOR WHERE MISTING EXISTS.

Glove Protection:

NEOPRENE OR NITRILE RUBBER GLOVES IF REPEATED OR PROLONGED SKIN CONTACT IS LIKELY.

Eye Protec tion:

SAFETY GLASSES IF THE METHOD OF USE PRESENTS THE LIKELIHOOD OF EYE CONTACT.

Other Protection:

WEAR GENERAL-DUTY WORK CLOTHES AND SHOES.

SECTION X - STORAGE AND HANDLING INFORMATION

Storage Temperature:    Indoors--> X	Outdoors—>	Heated-->	Refrigerated—>

Minimum. Temperature: 35°F    Maximum Temperature: 120°E*

Precautions to be Taken in Handling and Storing:

ALWAYS STORE MATERIAL IN ITS ORIGINAL CONTAINER. KEEP CONTAINER TIGHTLY CLOSED WHEN NOT IN USE. TO AVOID POSSIBLE EXPLOSIONS, AVOID WELDING ON PIPES OR TANKS WHICH HAVE CONTAINED THIS PRODUCT UNTIL THEY HAVE BEEN THOROUGHLY WASHED OUT WITH WATER. DO NOT RUN PUMPS WITH THE DISCHARGE OR SUCTION VALVES CLOSED; PUMP MUST BE ON CIRCULATION- EMPTY CONTAINERS MAY CONTAIN PRODUCT RESIDUES WHICH MAY EXHIBIT THE HAZARDS OF THE PRODUCT.

Otiier Precautions:

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE  LABEL BEFORE USING THE PRODUCT.

MATERIAL SAF^rY DATA SHEET: LIQUI -CELLERATE , O/G        Page: 3 SECTION X - STORAGE AND HANDLING INFORMATION (Continued)

SECTION XI - REGULATORY INFORMATION

Chemical Name	CAS dumber	Upper _% Linn, t

NITRATE COMPOUNDS	N/A	47

Those ingredients listed above are subject tc tine reporting requirements of 313 of Title III of the Superfund Amendments and Reauthorization Act J 986 and 40 CFR part 372.

se call 1-800-527-9919 for additional information if you are a California customer, s MSDS is not intended for users in the state of California.

SECTION XII - REFERENCES

1. THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS AND BIOLOGICAL EXPOSURE INDICES, ACGIH, 2002. 2 . OSHA PEL.

- VENDOR'S MSDS.

- REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFODisc, 2002.
ALL COMPONENTS IN THIS PRODUCT CAN BE FOUND IN THE CURRENT TSCA INVENTORY.

IRR: IRRITANT, FLAM/FLAMM: FLAMMABLE, COMB: COMBUSTIBLE,

CORK: CORROSIVE  CARC: CARCINOGENIC, TOX:TOXIC, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED,   COG: CLEVELAND OPEN CUP, PMCC: PENSKY -MARTIN CLOSED CUP, TCC:TAGLIABUE CLOSED CUP,

LEL:LOWER EXPLOSION LIMIT, UEL: UPPER EXPLOSION LIMIT, NFPA: NATIONAL FIRE PROTECTION ASSOCIATION, IARC: INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTP:NATIONAL TOXICOLOGY PROGRAM, OSHA: OCCUPATIONAL SAFETY 6 HEALTH ADMINISTRATION, ACGIH: AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLV: THRESHOLD LIMIT VALUE, PEL: PERMISSIBLE EXPOSURE LIMIT, STEL: SHORT-TERM EXPOSURE LIMIT, MLDlMILD, MOD: MODERATE, SEV:SEVERE, MUT:MUTAGENIC, ASPHYX: ASPHYXIANT, PNOS: PARTICULATES (INSOLUBLE) NOT OTHERWISE SPECIFIED, SDT: STANDARD DRAISE TEST, ORLrORAL,  IHL: INHALATION, HMN:HUMAN

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION,  HOWEVER, NO WARRANTY IS EXPRESSED OR IMP REGARDING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

MANTEK, DIVISION OF NCH CORP. assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the pr in a manner not recommended on the product label.  Users assume all risks associated with such unrecommended use, storage, or disposal of the

product.